Checklist: Clinical Protocol for Laser-Excised Specimen Handling

This checklist is designed for surgical oncology teams and pathology departments to ensure diagnostic integrity when utilizing laser technology for biopsies.

I. Intraoperative Preparation (Surgical Team)

- [ ] Langer’s Lines Identification: Map the skin tension lines at the biopsy site.

- [ ] Perpendicular Tension Applied: Manually spread the skin at a 90-degree angle to Langer’s lines.

  - Rationale: Creates a natural elliptical wound to minimize scarring and "dog ears."

- [ ] Surgical Tape Stabilization: Secure the spread skin with tape to maintain tension during computer-guided ablation.

- [ ] Computer Shape Alignment: Align the digital path (0.001" stroke width) with the physical boundary projected on the skin.

- [ ] Orientation Marking: Place a suture or permanent marker at the 12 o'clock position before final excision to assist the pathologist in orientation.

II. Post-Excision Stabilization (Nursing/Surgical Staff)

- [ ] Immediate Fixation: Submerge the specimen in 10% Neutral Buffered Formalin (NBF) immediately upon removal.

- [ ] Fixative Ratio Check: Ensure a minimum ratio of 10 parts formalin to 1 part tissue.

- [ ] Labeling: Clearly mark the container as "Laser Excision" to alert the pathologist to potential thermal artifacts.

III. Gross Examination (Pathology Lab)

- [ ] Differential Inking: Apply multiple ink colors to specific margins (e.g., Deep = Black, Superior = Green) to distinguish surgical boundaries from laser-charred areas.

- [ ] Thermal Zone Measurement: Document the thickness of the carbonized "char" layer in the gross report.

- [ ] Incision Quality Review: If the charring exceeds 0.5mm, flag the specimen for "Deep Levels" sectioning.

IV. Microscopic Evaluation (Pathologist)

- [ ] Artifact Identification: Distinguish between "streaming" nuclei (thermal stretching) and true cellular dysplasia.

- [ ] Coagulative Necrosis Exclusion: Disregard the narrow band of eosinophilic tissue at the immediate margin when grading tumor cells.

- [ ] Margin Status Reporting: Explicitly state if margins are:

  - Clear: No tumor within the defined margin.

  - Involved: Tumor present at the edge.

  - Obscured: Thermal artifact prevents definitive assessment.

- [ ] Ancillary Testing (If Needed): Order IHC markers (e.g., p16, cytokeratins) if thermal damage obscures standard H&E morphology.

V. Troubleshooting & Quality Control

- [ ] Feedback Loop: If artifacts are consistently high, notify the surgeon to:

  - Increase programmed speed (mm/s).

  - Switch to Super-Pulse mode.

  - Re-calibrate the laser focal point.

University of Washington | Surgical Oncology Lecture Series

Fleet Admiral Correo Hofstad, MD, MDiv, JD, JSD